Product Specification

ZH-C4

- I. Function Introduction:

- ZH-C4 belongs to full color series LED player, its functions are as follows:

- Adopt the same refresh technology as synchronous control system;

- Support cascading receiving cards, support any size screen;

- Support mobile APP (Android + iPhone) to send program;

- Support U-disk, Wi-Fi hot-spot (AP mode) and Internet (STA mode) to realize cluster management;

- Support remote cluster release management system;

- Offer SDK file;

- Support HD video hard decoding;

- Supports audio output, 3.5mm standard audio output interface;

- Support USB to play video and pictures directly;

- Support brightness automatic, timing adjustment;

- Support program play in sequence and timing;

- Support UDP network communication protocol;

- Support DHCP to obtain IP address automatically;

- Support emergency information insertion;

- Support weather forecast;

- Support 1 pcs HDMI output port;

- Support one-button full screen test;

- Support 2pcs video Windows to play simultaneously;

- Support manual switch, timing switch;

- Support log, screen status checking;

- Support multi-device play synchronously;

- Support cloud login dynamic verification, support grouping, classification, monitoring, verify and other permission to manage.

- II. Basic Function Parameters

| - System Parameters |  |
| --- | --- |
| - CPU | - 8-core A53, 1.5GHz |
| Operation System | Android 5.1 |
| System Storage | 8G (The system takes up about 2.5G) |
| Control Area | Loading Points: 2.0 million points Maximum Width: 4096，Maximum Height: 2160 |
| Support Resolution (Width*Height) | 960*2160 1012*2048 1080*1920 1152*1792 1296*1600 1480*1400 1518*1366 1620*1280 1800*1152 1920*1080 2048*1012 2304*900 2560*810 2592*800 2816*736 3072*672 3200*648 3264*632 3456*600 3600*576 3672*564 3840*540 4096*512 |
| Receiving card | All series ZH receiving card |
| Device Port |  |
| Synchronous input interface and quantity | Not support synchronous |
| Asynchronous input interface and quantity | Gigabit network*1,USB*2,WiFi*1 |
| Output port and quantity | Gigabit network*4, Audio*1, HDMI*1 |
| Media playback formats |  |
| program, window | Multi-program sequence, timing and other playback methods; arbitrary partition and overlay playback of windows |
| Video formats | AVI,F4V,WMV,MPG, RM/RMVB,MOV,VOB,MP4,FLV,TS,MKV and other common video formats; can support 2 video windows to play at the same time |
| Picture formats | BMP、JPG、PNG、GIF and so on |
| Audio formats | MPEG(MP3)、AAC and so on |
| Text document | Support software to edit single-line, multi-line, colorful text; support TXT documents import; support word, Excel, PPT, etc. of 2007 or later versions of office import |
| Other function | Support clock, watch face, timing, weather forecast (commercial grade), streaming media, etc. |
| Management and control |  |
| Software | PC：LEDPlayerV5；APP：LED Mobile Color; WEB |
| Program update communication | U disk export, plug and play; self-adaptive 100M/1000M network port direct connection or access to LAN to send; WiFi AP mode or STA mode control |
| FPGA file | Built-in many brand module FPGA file, can quickly debug the display |
| Brightness setting | Screen brightness can be set according to time |
| Remote controller | Play volume, signal, material, etc. can be controlled through the software remote control function |
| cluster mode | Support local server cluster, Support cloud server cluster |
| Safety |  |
| Local | Support local send password setting |

- III. Input/Output Port Arrangement and Definition

- Hardware Introduction:

- The Front Panel

- The Back Panel

| NO.: | Name: | Function: | Note: |
| --- | --- | --- | --- |
| 1 | Indicator | Power indicator | Red light always on |
| 2 |  | Operation indicator | Green light flashing regularly |
| 3 | Test button | Checking device information, test screen | 13 kinds of test mode |
| 4 | USB port | Read content, enlarge memory |  |
| 5 | USB port | Read content, enlarge memory |  |
| 6 | PC port | Link with PC, set parameter, send content |  |
| 7 | USB port | Hardware update |  |
| 8 | WiFi antenna port | Link with WiFi antenna |  |
| 9 | Sensor port | Connect with light sensor | Need to be customized |
| 10 | Reserved port | Reserved interface for backup | Need to be customized |
| 11 | HDMI port | HD HDMI output |  |
| 12 | Audio output port | Connect external active amplifier |  |
| 13 | Gigabit port | Link with receiving card |  |
| 14 | Gigabit port | Link with receiving card |  |
| 15 | Gigabit port | Link with receiving card |  |
| 16 | Gigabit port | Link with receiving card |  |
| 17 | Switcher | Power switcher |  |
| 18 | 220V port | 220V power input port |  |

- Working Condition:

| - Rated voltage (V) | - 220 | - Maximum | - 240 | - Minimum | - 100 |
| --- | --- | --- | --- | --- | --- |
| - Rated current (A) | - 0.15 | - Maximum | - 0.3 | - Minimum | - 0.1 |
| - Rated power consumption (W) | - 33 | - Maximum | - 50 | - Minimum | - 20 |
| - Extreme temperature(°C) |  | - Maximum | - 80 | - Minimum | - -40 |
| - Working environment temperature (°C) |  | - Maximum | - 80 | - Minimum | - -40 |
| - Working environment humidity (%) |  | - Maximum | - 95 | - Minimum | - 0 |

- IV.  Hardware Diagram: