Product Specification

ZH-Z12

- I. Product Introduction:

- The ZH-Z12 two-in-one multi-window video player is a product produced by ZH to meet the convenience of large LED screen installation. It supports HDMI, DP and other high-definition signals four-window arbitrary roaming input; support max pixel 4096*2160@60Hz,12 PCS Gigabit Ethernet port output, single device supports pixels: Max width 16384,or Max Height 4320. Adopt a new communication architecture, high refresh, high brightness, new grayscale scanning engine, nanosecond synchronization, and communication can be maintained smoothly under bad communication conditions, allowing users to experience a more colorful and better LED world!

- II. Functional Features：

- 1. Input interfaces, 1pcs HDMI 2.0, 2pcs HDMI 1.4, 1pcs DP, 1pcs VGA;

- 2. Input resolution: Max pixel is 4096*2160@60Hz,supports arbitrary setting of resolution;

- 3. Single card Max loading: 7.8 million points, Max Width 16384, Max Height: 4320;

- 4. Support arbitrary switching of video sources, splicing and zooming input images according to the resolution of the display screen;

- 5. Support four windows roaming freely;

- 6. Support dual 4K@60HZ input;

- 7. Support multi-machine 4K splicing and cascading;

- 8. Support audio input and output;

- 9. Support brightness and color temperature adjustment;

- 10. 12 PCS Gigabit Ethernet port output, support arbitrary splicing up and down, left and right;

- 11. Support hot backup: two network ports in the same group for hot backup and multi-card hot backup;

- 12. Support two debugging methods of device panel and PC;

- 13. Support low brightness and high gray;

- 14. Support ZH full series of receiving cards, multi-function cards, and fiber optic transceivers;

- III. Topology Diagram:

- IV.  Hardware Introduction:

The Front Panel

| Switch | The power switch |
| --- | --- |
| LCD | Show the device status and operation interface |
| Knob | Pressing:To enter or confirm; Rotation: To move the cursor or modify parameters |

| Button Part Manual |  |  |  |
| --- | --- | --- | --- |
| SOURCE |  | FUNCTION |  |
| HDMI/1 | HDMI 1/number 1 | WIN/4 | Multi-window settings/number 4 |
| HDMI/2 | HDMI 2/number 2 | PART/5 | Partial and Full switch/number 5 |
| HDMI/3 | HDMI 3/number 3 | MENU | Enter the menu advanced setting mode |
| DP/6 | DP/number 6 | FN/9 | Compound key/number 9 |
| VGA/7 | VGA/number 7 | SCENE/0 | Scene shortcut key/number 0 |
| EXT/8 | EXT/number 8 | ESC | Back |

The Back Panel

| CONFIG |  | Single Input |  | Single Output |  |
| --- | --- | --- | --- | --- | --- |
| USB_IN | System setting port | HDMI_IN*3 | HDMI input | LED*12 | LAN output 1--12 |
| USB_OUT | Device cascading interface port | 4K DP_IN | DP input | AUDIO_OUT | Audio output |
| U-DISK | Reserved port | VGA_IN | VGA input |  |  |
| RS232_IN | Central control serial port | AUDIO VGA_IN | VGA audio input |  |  |

Input:

| Port | Quantity | Resolution |
| --- | --- | --- |
| HDMI 1.4 | 2 | Stander EIA/CEA-861 |
| HDMI 2.0 | 1 | Stander EIA/CEA-861, match HDMI-2.0 stander, support HDCP2.2 |
| VGA | 1 | Support 1920*1200/60Hz and so on, match VESA stander resolution |
| DP | 1 | Match DP-1.2 stander, support HDCP1.3 |
| AUDIO | 1 | 3.5mm stander audio input port |
| USB | 1 | Standard USB2.0port |

| Port | Quantity | Resolution |
| --- | --- | --- |
| LED | 12 | Standard Gigabit Ethernet port, support ZH V5 all types of receiving cards, support arbitrary splicing up and down or left and right |
| AUDIO | 1 | 3.5mm standard audio output port |

Output:

Chassis parameters:

| Power Input | AC 100~240V，50/60Hz,not more than 1A |
| --- | --- |
| Temperature | -20~45℃ |
| Size | 482*327.73*72(mm,L*W*H) |
| Weight | ≈4.3KG |

- VI. Hardware Diagram:

Unit: mm