Product Specification

ZH-Z12R

- I. Product Introduction:

- ZH-Z12R two-in-one multi-window video player is a product produced by ZH to meet the convenience of large LED screen installation. It supports HDMI, DP and other high-definition signal input; support max pixel 4096*2160@60Hz, 12 PCS Gigabit Ethernet port output, single device supports pixels: Max width 8192,or Max Height 4096. Adopt new communication architecture, high refresh, high brightness, new grayscale scanning engine, nanosecond synchronization, and communication can be maintained smoothly under bad communication conditions, allowing users to experience a more colorful and better LED world!

- II. Functional Features：

- Input interfaces, 1pcs HDMI 2.0, 2pcs 1.4, 1pcs DP, 1pcs VGA;

- 2. Input resolution: Max pixel is 4096*2160@60Hz,supports arbitrary setting of resolution;

- 3. Single card Max loading: 7.8 million points, Max Width 8192, Max Height: 4096;

- 4. Support arbitrary switching of video sources, splicing and zooming input images according to the resolution of the display screen;

- 5. Support four-picture PIP, PBP;

- 6. Support output screen preview;

- 7. Support audio input and output;

- Support brightness and color temperature adjustment

- 12 PCS Gigabit Ethernet port output, support arbitrary splicing up and down, left and right;

- Support hot backup: two network ports in the same group for hot backup and multi-card hot backup;

- Support low brightness and high gray;

- Support ZH full series of receiving cards, multi-function cards, and fiber optic transceivers;

- Support engineering lock.

- III. Topology Diagram:

- IV.  Hardware Introduction:

The Front Panel

| Switch | The power switch |
| --- | --- |
| USB Port | PC software setting port |
| LCD | Displays detailed prompts for user |
| Knob | Adjust the LED screen resolution and confirm |

| Button Part Manual |  |  |  |
| --- | --- | --- | --- |
| SOURCE |  | FUNCTION |  |
| HDMI/1 | HDMI 1/number 1 | WIN/4 | Multi-window settings/number 4 |
| HDMI/2 | HDMI 2/number 2 | PART/5 | Partial and Full switch/number 5 |
| HDMI/3 | HDMI 3/number 3 | MENU | Enter the menu advanced setting mode |
| DP/6 | DP/number 6 | FN/9 | Compound key/number 9 |
| VGA/7 | VGA/number 7 | SCENE/0 | Scene shortcut key/number 0 |
| EXT/8 | EXT/number 8 | ESC | Back |

The Back Panel

| CONFIG |  | Single Input |  | Single Output |  |
| --- | --- | --- | --- | --- | --- |
| USB_IN | System setting | HDMI_IN*3 | HDMI input | LED*12 | LAN output 1--12 |
| USB_OUT | Device cascade debugging interface | DP_IN 4K | DP input | AUDIO_OUT | Audio output |
| ANTENNA | 4G antenna port | VGA_IN | DP input | HDMI_MONITOR | Screen preview |
| SENSOR | Sensor(optional) | AUDIO VGA_IN | VGA audio input |  |  |

Input:

| Port | Quantity | Resolution |
| --- | --- | --- |
| HDMI 1.4 | 2 | Stander EIA/CEA-861 |
| HDMI 2.0 | 1 | Stander EIA/CEA-861, match HDMI-2.0 stander, support HDCP2.2 |
| VGA | 1 | Support 1920*1200/60Hz and so on, match VESA stander resolution |
| DP | 1 | Match DP-1.2 stander, support HDCP1.3 |
| AUDIO | 1 | 3.5mm stander audio input port |
| USB | 1 | Standard USB2.0port |

Output:

| Port | Quantity | Resolution |
| --- | --- | --- |
| LED | 12 | Standard Gigabit Ethernet port, support ZH V5 all types of receiving cards, support arbitrary splicing up and down or left and right |
| AUDIO | 1 | 3.5mm standard audio output port |

Chassis parameters:

| Power Input | AC 100~240V，50/60Hz,not more than 1A |
| --- | --- |
| Temperature | -20~45℃ |
| Size | 482*281.5*66.5(mm,L*W*H) |
| Weight | 4KG |

- VI. Hardware Diagram:

Unit: mm