Product Specification

ZH-Z24

- I. Product Introduction:

- The ZH-Z24 two-in-one multi-window video player is a product produced by ZH to meet the convenience of large LED screen installation. It supports HDMI, DP and other high-definition signal input; can receive up to 4096*2160@60Hz pixel high-definition digital signal, 24 PCS Gigabit Ethernet port output, single device supports pixels: Max width 16384,or Max Height 4096. Adopt a new communication architecture, high refresh, high brightness, new grayscale scanning engine, nanosecond synchronization, and communication can be maintained smoothly under bad communication conditions, allowing users to experience a more colorful and better LED world!

- II. Functional Features：

- Input interfaces, 2pcs HDMI 2.0, 2pcs 1.4, 1pcs DP, 1pcs VGA;

- Input resolution: Max pixel is 4096*2160@60Hz,supports arbitrary setting of resolution;

- Single card Max loading: 15.6 million points, Max Width 16384, Max Height: 4096;

- Support arbitrary switching of video sources, splicing and zooming input images according to the resolution of the display screen;

- Support four wis roaming freely;

- Support audio input and output;

- Support brightness and color temperature adjustment

- 24 PCS Gigabit Ethernet port output, support arbitrary splicing up and down, left and right;

- Support hot backup: two network ports in the same group for hot backup and multi-card hot backup;

- Support low brightness and high gray;

- Support ZH full series of receiving cards, multi-function cards, and fiber optic transceivers;

- III. Topology Diagram:

- IV.  Hardware Introduction:

The Front Panel

| Switch | The power switch |
| --- | --- |
| USB Port | PC software setting port |
| LCD | Displays detailed prompts for user |
| Knob | Adjust the LED screen resolution and confirm |

| Button Part Manual |  |  |  |
| --- | --- | --- | --- |
| SOURCE |  | FUNCTION |  |
| HDMI/1 | HDMI 1/number 1 | WIN/4 | Multi-window settings/number 4 |
| HDMI/2 | HDMI 2/number 2 | PART/5 | Partial and Full switch/number 5 |
| HDMI/3 | HDMI 3/number 3 | MENU | Enter the menu advanced setting mode |
| DP/6 | DP/number 6 | FN/9 | Compound key/number 9 |
| VGA/7 | VGA/number 7 | SCENE/0 | Scene shortcut key/number 0 |
| EXT/8 | EXT/number 8 | ESC | Back |

The Back Panel

| CONFIG |  | Single Input |  | Single Output |  |
| --- | --- | --- | --- | --- | --- |
| USB_IN | System setting | HDMI_IN*4 | HDMI input | LED*24 | Output LAN port 1--24 |
| USB_OUT | Device cascade debugging interface | DP_IN 4K | DP input | AUDIO_OUT | Audio port |
| U-DISK | Update port | VGA_IN | DP input |  |  |
| UART_IN | Intelligent console interface | AUDIO VGA_IN | VGA Audio input |  |  |
| External Interface |  |  |  |  |  |
| A1，A2 | Antenna port |  |  |  |  |
| S1，S2 | Sensor (operational) |  |  |  |  |

Input:

| Port | Quantity | Resolution |
| --- | --- | --- |
| HDMI 1.4 | 2 | Stander EIA/CEA-861 |
| HDMI 2.0 | 2 | Stander EIA/CEA-861, match HDMI-2.0 stander, support HDCP2.2 |
| VGA | 1 | Support 1920*1200/60Hz and so on, match VESA stander resolution |
| DP | 1 | Match DP-1.2 stander, support HDCP1.3 |
| AUDIO | 1 | 3.5mm stander audio input port |
| USB | 1 | Standard USB2.0port |

| Port | Quantity | Resolution |
| --- | --- | --- |
| LED | 24 | Standard Gigabit Ethernet port, support ZH V5 all types of receiving cards, support arbitrary splicing up and down or left and right |
| AUDIO | 1 | 3.5mm standard audio output port |

Output:

Chassis parameters:

| Power Input | AC 100~240V，50/60Hz,not more than 1A |
| --- | --- |
| Temperature | -20~45℃ |
| Size | 482*281.5*66.5(mm,L*W*H) |
| Weight | 6.5KG |

- VI. Hardware Diagram:

Unit: mm