Product Specification

ZH-Z6E

- I. Product Introduction:

ZH-Z6E is one three-in-one product developed for LED large-screen display systems, integrating asynchronous, high-definition video processors and sending cards. It adopts industry-leading lossless zoom technology, supports high-definition HDMI/DVI input and standard Gigabit Ethernet port output, and provides customers with a synchronous LED large-screen high-definition display solution; new asynchronous expansion card is added to meet various needs of customers in different scenarios; The integrated software simplifies the customer's operation process; the standard 1U chassis makes engineering installation more convenient and quicker, and truly achieves the purpose of multi-scenario and minimal operation. ZH-Z6E supports one-key switching between partial and full-screen modes, and one-key switching between synchronous and asynchronous modes. The screen transition is smooth and stable. ZH-Z6E supports online upgrades to ensure that customers upgrade new functions from time to time.

- II. Functional Features

ZH-Z6E is a full-color series player control system, its functions are as follows:

- Use the same refresh technology as the synchronization system;

- Support cascaded receiving card, realize the support of any size screen;

- SDK development kits are provided for all series;

- Support HD video hard decoding;

- Support audio sync audio input and output, 3.5mm standard audio output interface;

- Support U disk to play videos and pictures directly;

- Support 2 HDMI inputs, 1 VGA input, 1 HDMI output;

- Support synchronous, asynchronous local and remote one-key switching;

- Support one-key switching between partial and full screens in synchronous mode;

- Support picture zoom function;

- Support manual switch, timing switch;

- Support extended sensor and other functional interfaces;

- Support U disk, network cable to enable wireless WiFi hotspot (AP mode) and access to LAN (STA mode) to achieve cluster management;

- Support UDP network communication protocol;

- Support DHCP to automatically obtain IP address;

- Support one-key whole screen test;

- Support simultaneous playback of multiple video windows;

- Support EDID setting;

- Support brightness timing adjustment;

- Support Mobile (Android + Apple) software sending;

- Support local server and remote server to publish content;

- Support program sequence, timing and other playback methods;

- Support emergency information insertion;

- Support weather forecast, web pages, streaming media, etc.;

- Support standard databases such as json and mysyq;

- Support docking of data acquisition instrument, PLC, etc.;

- Support log, screen status query;

- Supports simultaneous playback of multiple devices;

- Support cloud login dynamic verification, support grouping, classification, monitoring, audit and other authority management.

- III. Function Parameters

System Parameter

| - CPU | - Eight-core A53，1.5GHz |
| --- | --- |
| - Control system | - Android 5.1 |
| - Storage | - 8G（system about 2.5G） |
| - Loading area | - 3.9 million - Max W: 8192, Max H: 2160 |

Communication and Interface

- Support WIFI;

- Support USB;

- Support 100 m / 1000 m network adaptive;

- Support server cluster;

Load Parameters

- Support Gigabit to cascade receiving card, maximum support 3.9 million points;

- IV.  Hardware Introduction:

The Front Panel

Switch

Button

Knob

LCD

| Switch | The power switch |
| --- | --- |
| LCD | Displays detailed prompts for user |
| Knob | Adjust the LED screen resolution and confirm |

| Button Part Manual |  |  |  |
| --- | --- | --- | --- |
| INPUT |  | FUNCTION |  |
| HDMI 1 | HDMI 1 signal input button | PART/局部 | Partial/full screen switching |
| HDMI 2 | HDMI 2 signal input button | MENU/菜单 | Enter advanced settings mode |
| VGA | VGA signal input button | OK/确认 | Select conform button |
| BOX | Async play selection key | Fn | Compound key |
| R | Reserved button | SCENE/场景 | The call button for default scenes |
|  |  | BACK/back | Back to the previous menu/lock function |

The Back Panel

Control Port

Power Port

Output Port

Input Port

| CONFIG/Control Port |  | INPUT/Signal Input Port |  | OUTPUT/Signal Output Port |  |
| --- | --- | --- | --- | --- | --- |
| SENSOR1 | Extended sensor interface 1 | USB | Update content, expand memory | AUDIO | Audio output port |
| SENSOR2 | Extended sensor interface 2 | HDMI 1_IN | HDMI input port | MONITOR | HDMI output port |
| ANTENNA | Remote controller antenna port | HDMI 2_IN | HDMI input port | LED1/LED2 | Output network port |
| USB_CTR | System test port | VGA_IN | VGA input port |  |  |
|  |  | VGA AUDIO | VGA audio input port |  |  |

| EXTEND |  |
| --- | --- |
| Test button | Check version, multiple test modes |
| LAN port | Update the content |
| USB port | System firmware update (non-professionals should not operate it) |
| USB port | Update content, enlarge storage |
| WiFi antenna port | Link with WIFI, enhance WIFI signal |
| Audio input port | Asynchronous audio output, connect to power amplifier |
| Indicator light | Operation indicator light (normal state: red light is on, green light is flashing) |
| SIM card port | 4G data card port |
| 4G antenna port | Link with 4G antenna, enhance 4G signal |

Working conditions

| Rated voltage (V) | 220 | Max | 100 | Min | 240 |
| --- | --- | --- | --- | --- | --- |
| Rated current (A) | 0.1 | Max | 0.15 | Min | 0.07 |
| Rated power consumption (W) | 20 | Max | 25 | Min | 15 |
| Limit temperature (℃) |  | Max | 80 | Min | -40 |
| Working environment temperature (℃) |  | Max | 80 | Min | -40 |
| Working environment humidity (%) |  | Max | 95 | Min | 0 |

- VI. Hardware Diagram:

Unit: mm